- How to install an OmniPCXOffice-CPU

- Introduction

The aim of this short document is to give a practical and simple example of how to install a complete software including IA, VoIP and software licenses using LOLA and PM5 with Ethernet direct connections.

- References

[1] Software download for ALIZE-CPU & - LoLa R300	Ref :3AC_91000_5034_FAAFA_01

- Procedure 1 (Software on the Hard Disk)

Action on  PM5 are in green, actions on LOLA are in purple and actions on CPU are in blue.

- Make sure that the software to install is stored on the PC, either locally (on Hard disk) or on a CD-Rom. If the software is stored on the network then it is necessary to follow the explanations of Procedure 2 (see in bottom)

- Set the DIP switch 3 to OFF on the CPU board.

- Plug the CPU board (without starting it).

- Start LOLA.

- Choose a software to install :

- if you do not have safeguard of a configuration (xxx.lol) :

- select the delivery that you want to install

- select the country that you want to install

- if delivery is 300, select the CTI licence  that you want to install

- if delivery is 300 ,select the Main licence that you want to install

- select the service that you want to install (for example IA , VOIP)

- if you have safeguard of a configuration (xxx.lol) :

- with the menu File/Open, select the file xxx.lol

- if you want, modify the country or the licence or the service

- Click Next

- Choose the type installation according to what you want to do (for example for a simple install choose : installation)

- Click Next

- Connect the Ethernet crossed cable between CPU and PC (LAN sockets).

- Click Yes (DIP Switch and Ethernet  crossed cable  is OK)

- LOLA is waiting for Requests.

- Start the CPU (switch on).

After a couple of minutes the connection is established  and the download starts ( it then takes about 5 min.).

- In the principal windows one has the course of the various stages of remote loading

- When the traffic is finish then the message of end of download and disconnection appears.

- Switch off the CPU and wait until the led stops blinking.

- Unplug the CPU and set the DIP switch 3 to ON.

- Put the CPU back in a cabinet and switch it on; the CPU starts twice.

If the CPU board is meant to be plug in a cabinet with a different hardware configuration, proceed as follows:

- Program a manual Cold Reset (menu System particularities/system reset) : that means if you switch the CPU off and on again it will restart with default configuration.

- Switch off the CPU and wait until the led stops blinking.

- Unplug the CPU and plug it in the customer’s system.

- The system will start cold with Software licenses and Download items previously downloaded.

- Procedure 2  (Software on the network)

Action on  PM5 are in green, actions on LOLA are in purple and actions on CPU are in blue.

- Make sure that the software to install is stored on the network.

- Set the DIP switch 3 to OFF on the CPU board.

- Plug the CPU board (without starting it).

- Start LOLA.

- Choose a software to install :

- if you do not have safeguard of a configuration (xxx.lol) :

- select the delivery that you want to install (on the Network)

- select the country that you want to install

- if delivery is 300, select the CTI licence  that you want to install

- if delivery is 300 ,select the Main licence that you want to install

- select the service that you want to install (for example IA , VOIP)

- if you have safeguard of a configuration (xxx.lol) :

- with the menu File/Open, select the file xxx.lol

- if you want, modify the country or the licence or the service

- Click Next

- Choose the type installation according to what you want to do (for example for a simple install choose : installation)

- Click Next

- Click Yes (DIP Switch and Ethernet  crossed cable  is OK)

- Wait a few minutes that LOLA copy the delivery on the Hard Disk

- LOLA is waiting for Requests.

- Connect the Ethernet crossed cable between CPU and PC (LAN sockets).

- Start the CPU (switch on).

After a couple of minutes the connection is established  and the download starts ( it then takes about 5 min.).

- In the principal windows one has the course of the various stages of remote loading

- When the traffic is finish then the message of end of download and disconnection appears.

- Switch off the CPU and wait until the led stops blinking.

- Unplug the CPU and set the DIP switch 3 to ON.

- Put the CPU back in a cabinet and switch it on; the CPU starts twice.

If the CPU board is meant to be plug in a cabinet with a different hardware configuration, proceed as follows:

- Program a manual Cold Reset (menu System particularities/system reset) : that means if you switch the CPU off and on again it will restart with default configuration.

- Switch off the CPU and wait until the led stops blinking.

- Unplug the CPU and plug it in the customer’s system.

- The system will start cold with Software licenses and Download items previously downloaded.

- END